Ministry of Education and Research of the Republic of Moldova

Technical University of Moldova

Department of Software and Automation Engineering

REPORT

Laboratory work No. 3.1

Discipline: Embedded Systems

Elaborated:   Berco Andrei, FAF - 221

Checked:	asist. univ., Martiniuc A.

Chișinău 2025

Analysis of the Situation in the Field

- Description of the Technologies Used and Application Context

Embedded systems are fundamental in modern applications, where real-time data acquisition and processing are critical. Microcontrollers (MCUs) equipped with analog and digital sensors enable precise environmental monitoring, automation, and control systems.

This laboratory work focuses on signal acquisition from sensors (analog or digital), processing the data, and displaying it on an LCD or Serial terminal using FreeRTOS for task scheduling and STDIO for reporting. The system demonstrates modularity, real-time monitoring, and efficient task management—essential in industrial automation, IoT, and smart sensing applications.

- Overview of the Hardware and Software Components Used

Microcontroller (ESP32/STM32/Arduino with FreeRTOS) – Central unit for sensor data acquisition, processing, and task scheduling.

- Sensor (Analog/Digital) – Selected from 37Sensors (e.g., temperature sensor, motion sensor, etc.)

- Breadboard & Jumper Wires – For prototyping and interconnections.

- Power Supply (USB/Battery) – Provides stable voltage to the MCU and peripherals.

Software Components

- Development Environment (PlatformIO/Arduino IDE/STM32CubeIDE) – For coding, compiling, and flashing the firmware.

- FreeRTOS – Manages task scheduling (vTaskDelay, vTaskDelayUntil) for periodic sensor readings and display updates.

- STDIO Library (printf, scanf) – Facilitates Serial communication and LCD output for reporting sensor data.

- Sensor-Specific Libraries (e.g., DHT.h for temperature, Wire.h for I2C sensors) – Enables seamless sensor interfacing.

- System Architecture Explanation and Solution Justification

The system follows a modular FreeRTOS-based architecture, where tasks are scheduled for:

Sensor Data Acquisition – Reads analog/digital signals at fixed intervals.

Signal Processing – Scales raw data into physical values (e.g., °C, lux).

Data Display – Outputs processed data to LCD/Serial every 500ms.

Why This Solution?

- Real-Time Performance – FreeRTOS ensures precise timing for sensor sampling.

- Modularity – Separates acquisition, processing, and display for scalability.

- Efficient Resource Use – Non-blocking delays prevent CPU overload.

- Flexibility – Supports multiple sensor types (analog/digital).

- Case Study: Sequential Task Execution in Embedded Systems

#### Context and Necessity

#### Many embedded applications (e.g., weather stations, smart agriculture) require periodic sensor readings with minimal latency. This case study demonstrates a FreeRTOS-based approach for reliable signal acquisition

#### Practical Implementation

- Task 1: Sensor Read

  - Analog sensors: analogRead() → scaling (e.g., Vout = (ADC_val / 4095) * 3.3V).

  - Digital sensors: I2C/SPI communication (e.g., BME280.readTemperature()).

  - Uses vTaskDelayUntil() for consistent sampling intervals.

- Task 2: Data Processing

  - Applies calibration/formulas (e.g., Temp = (Vo * 100) / 1024).

  - Stores results in a global struct for other tasks.

- Task 3: Display & Reporting

  - Prints formatted data via printf() every 500ms:

#### Extending the Case Study

- Wireless Transmission – Integrate Wi-Fi (ESP32) to send data to a cloud dashboard.

- Multi-Sensor Fusion – Add more sensors (CO₂, pressure) with independent tasks.

- Energy Optimization – Use vTaskSuspend() in battery-powered scenarios.

#### Benefits and Impact

- Structured Timing – FreeRTOS ensures no task starvation.

- Scalability – Easy to add more sensors/tasks.

- Debugging-Friendly – STDIO provides real-time logs.Scalability – Can be integrated with wireless communication, IoT frameworks, or real-time monitoring systems.

This case study highlights the importance of structured scheduling in embedded applications, demonstrating how sequential execution models enhance reliability and efficiency in task management.

Design

- Architectural Sketch and Component Interconnection

The system consists of:

- MCU (ESP32/Arduino) – Core processing unit.

- Sensor – Connected via:

- Analog: Direct to ADC pin.

- Digital: I2C (SCL/SDA) or SPI (MISO/MOSI/SCK).

- Serial Monitor – For debugging (printf output).

Figure 2.1 Component diagram

Figure 2.2 Component scheme

Figure 2.3 Electrical scheme

- Scheme bloc and algorithm

To understand the system's behavior, a Flowchart and a Finite State Machine (FSM) are used.

Flowchart – Serial Command Processing
(A Flowchart diagram illustrating the cycle: command reception → processing → execution → user feedback)

Figure 2.4 Flowchart

Explanation:

- Initialization: Hardware (sensor, Serial) and FreeRTOS tasks are set up.

- Acquisition Task:

  - Reads sensor data (sensor_read_distance).

  - Updates shared signals (signal_manager_update).

  - Uses vTaskDelayUntil for precise 100ms intervals.

- Display Task:

  - Retrieves signals (signal_manager_get_signals).

  - Prints formatted report (printReport) every 500ms.

Figure 2.4 FSM diagram

States:

- SYSTEM_IDLE: Default state (no active measurements).

- SYSTEM_MEASURING: Valid sensor data received (distance ≥ 0).

- SYSTEM_ERROR: Invalid data (distance < 0 or sensor failure).

Transitions:

- Valid Data: Moves from IDLE to MEASURING.

- Error Condition: Invalid distance triggers ERROR state.

- Recovery: Manual reset or valid data resumes MEASURING/IDLE.

- Modular implementation

For better project organization, a modular architecture was used, dividing functionalities into separate files.

Figure 3.1 Project organization

The project is well-structured, separating functionality into different modules using a layered approach. The organization follows best practices for embedded systems and modular programming.

#### Code Functionality Overview:

- main.cpp

Figure 3.1: Flowchart for main.cpp

Description

- Initialization:

  - Sets up Serial communication (Serial.begin(115200).

  - Redirects printf to Serial via fdev_setup_stream.

- Hardware Setup:

  - Calls sensor_init() (configures HC-SR04 pins).

  - Initializes signal manager (signal_manager_init() creates mutex).

- RTOS Tasks:

  - Creates two tasks:

    - acquisitionTask (priority 2, 128B stack).

    - displayTask (priority 1, 256B stack).

  - Starts FreeRTOS scheduler (vTaskStartScheduler).

- Loop: Empty (scheduler handles execution).

- sensor.cpp

Figure 3.2: Flowchart for sensor.cpp

Description

- sensor_init():

  - No explicit setup (handled by NewPing library).

- sensor_read_distance():

  - Triggers ultrasonic pulse (sonar.ping_cm()).

  - Measures echo time → converts to cm.

  - Validates reading:

    - Returns MAX_DISTANCE (400cm) if no echo.

    - Else returns actual distance (0–400cm).

- signal_manager.cpp

Figure 3.4: Flowchart for signal_manager.cpp

Description

- signal_manager_init():

  - Creates mutex (xSemaphoreCreateMutex) to protect shared SystemSignals.

- signal_manager_update():

  - Locks mutex → updates distance, count, timestamp.

  - Calls updateSystemState() to set state and errorCode.

  - Releases mutex.

- signal_manager_get_signals():

  - Safely copies SystemSignals using mutex.

- updateSystemState():

  - Sets state based on distance:

    - ERROR if distance < 0.

    - MEASURING otherwise (with error code 0).

- display.cpp

Figure 3.5: Flowchart for display.cpp

Description

- printReport():

  - Formats and prints SystemSignals via printf:

    - State (IDLE/MEASURING/ERROR).

    - Distance, measurement count, error code (if any).

- displayTask():

  - Starts with 50ms offset (to avoid collision with acquisition task).

  - Every 500ms:

    - Retrieves signals (signal_manager_get_signals).

    - Calls printReport().

Results

Figure 3.6: Physical result

Figure 3.7: Console result

Conclusions

After completing this laboratory work, the following results were achieved:

A functional sensor data acquisition and display system was successfully implemented, enabling real-time measurement, signal processing, and reporting using FreeRTOS and STDIO.

- The application correctly acquires ultrasonic sensor data, processes it into meaningful measurements (distance in cm), and displays the results on Serial and/or LCD at fixed intervals.

- A modular architecture was employed, separating functionalities into distinct tasks: sensor acquisition (100ms), signal management (mutex-protected), and display (500ms).

- The system was tested on a microcontroller platform (e.g., Arduino with FreeRTOS) and validated for real-time performance and data accuracy.

- Error handling was integrated to detect invalid sensor readings (e.g., negative distances), triggering an error state for robustness.

System Performance Analysis

The system demonstrated stable and efficient operation, with reliable sensor sampling and reporting. Key performance aspects include:

- Timing Precision:

  - Sensor data is acquired every 100ms using vTaskDelayUntil() for minimal jitter.

  - Display updates occur every 500ms without blocking acquisition.

- Data Integrity:

  - Shared signals (e.g., distance, state) are protected by a mutex, preventing race conditions.

  - Invalid readings (e.g., out-of-range values) automatically trigger an error state.

- Scalability:

  - The modular design allows easy integration of additional sensors (e.g., temperature, humidity) or output devices (e.g., Wi-Fi modules).

Identified Limitations

Despite its functionality, the system has areas for improvement:

- Sensor Error Recovery – The system detects errors but lacks an auto-recovery mechanism (e.g., retry logic).

- Limited Output Channels – Data is only displayed on Serial; adding LCD or wireless (Wi-Fi/MQTT) would enhance usability.

- Fixed Sampling Rate – The 100ms interval is hardcoded; a user-configurable rate (e.g., via buttons) would improve flexibility.

- Power Efficiency – No low-power modes are implemented, which could be critical for battery-operated deployments.

Impact of the Technology in Real-World Applications

This project demonstrates principles applicable to:

- Industrial Monitoring:

  - Ultrasonic sensors measure tank levels or object proximity, with FreeRTOS ensuring real-time data logging.

- Smart Agriculture:

  - Soil moisture or light sensors could replace the ultrasonic module, with data transmitted to a cloud dashboard.

- Home Automation:

  - The system could be extended to control actuators (e.g., lights, alarms) based on sensor thresholds.

- IoT Prototyping:

  - Adding an ESP32 Wi-Fi module would enable remote monitoring via MQTT/HTTP.

Improvement Suggestions

To enhance the system, consider:

- Dynamic Sampling Rates – Allow runtime adjustment (e.g., via Serial commands) for adaptive sensing.

- Wireless Integration – Use ESP-NOW or LoRa for remote data transmission.

- Advanced Error Handling – Implement sensor calibration or auto-retry on failure.

- Power Optimization – Integrate FreeRTOS tickless idle mode for battery savings.

- Multi-Sensor Support – Expand the signal manager to handle multiple sensor inputs (e.g., temperature + distance).

By addressing these points, the system could evolve into a versatile embedded platform for real-world IoT and automation applications.

Final Remarks

This lab successfully achieved its objectives, providing a foundation for real-time sensor systems with FreeRTOS. The modular design, mutex-protected data, and structured reporting make it a reproducible template for future embedded projects.

Next Steps:

- Integrate Wi-Fi (ESP32) for cloud connectivity.

- Develop a GUI dashboard (e.g., Node-RED) for visualization.

- Benchmark power consumption and optimize for low-energy scenarios.

### Note on AI Tool Usage

During the drafting of this report, the author used ChatGPT for generating and structuring the content. The resulting information was reviewed, validated, and adjusted according to the requirements of the laboratory work, ensuring technical accuracy and clarity of explanations.
The use of this AI tool was aimed at structuring and optimizing the presentation of information without replacing personal analysis and understanding of the subject.

Bibliography

- Official Arduino Documentation

  - Arduino Reference – Serial Communication
https://www.arduino.cc/reference/en/#communication

  - Arduino Mega 1280 Pinout & Datasheet
https://docs.arduino.cc/hardware/mega-1280

- PlatformIO Official Documentation

  - PlatformIO for Arduino Development
https://docs.platformio.org/en/latest/platforms/atmelavr.html

- TUM Courses

  - Introducere în Sistemele Embedded și Programarea Microcontrolerelor

  - Principiile comunicației seriale și utilizarea interfeței UART

Appendix

- GitHub: https://github.com/KaBoomKaBoom/ES_Labs.git

- Main.cpp

/*

* Ultrasonic Sensor Data Acquisition and Display using FreeRTOS

*

* Main file - Program initialization and entry point

*/

#include <Arduino_FreeRTOS.h>

#include <stdio.h>

#include "sensor.h"

#include "signal_manager.h"

#include "display.h"

// Task handles

TaskHandle_t acquisitionTaskHandle;

TaskHandle_t displayTaskHandle;

// Required for printf redirection to Serial

int serialPutchar(char c, FILE *) {

Serial.write(c);

return c;

}

// Setup printf

FILE serial_stdout;

void setup() {

// Initialize serial communication

Serial.begin(115200);

// Set up printf to output to Serial

fdev_setup_stream(&serial_stdout, serialPutchar, NULL, _FDEV_SETUP_WRITE);

stdout = &serial_stdout;

// Print startup message

printf("\nUltrasonic Sensor System Starting...\n");

// Initialize sensor

sensor_init();

// Initialize signal manager

signal_manager_init();

// Create FreeRTOS tasks

xTaskCreate(

acquisitionTask,          // Task function

"AcquisitionTask",        // Task name

128,                      // Stack size

NULL,                     // Parameters

2,                        // Priority (higher number = higher priority)

&acquisitionTaskHandle    // Task handle

);

xTaskCreate(

displayTask,              // Task function

"DisplayTask",            // Task name

256,                      // Stack size (larger for display task due to printf)

NULL,                     // Parameters

1,                        // Priority (lower than acquisition task)

&displayTaskHandle        // Task handle

);

// Start the scheduler

vTaskStartScheduler();

}

void loop() {

// Empty, as FreeRTOS takes control

}

- Signal_manager.cpp

/*

* signal_manager.cpp

*

* Implementation file for signal and state management

*/

#include "signal_manager.h"

#include "sensor.h"

#include <string.h>

// Global system signals

static SystemSignals signals = {0, SYSTEM_IDLE, 0, 0, 0};

// Mutex for protecting access to shared signals

static SemaphoreHandle_t signalsMutex;

/**

* Update system state based on sensor data

*/

static void updateSystemState(SystemSignals *signals) {

// Check for sensor errors

if (signals->distance < 0) {

signals->state = SYSTEM_ERROR;

signals->errorCode = 1; // Error code 1: Invalid distance reading

}

// Check if object is too close (less than 10cm)

else if (signals->distance < 10) {

signals->state = SYSTEM_MEASURING;

signals->errorCode = 0; // No error

}

// Normal operation

else {

signals->state = SYSTEM_MEASURING;

signals->errorCode = 0; // No error

}

}

void signal_manager_init() {

// Create mutex for signals

signalsMutex = xSemaphoreCreateMutex();

// Initialize signals

signals.distance = 0;

signals.state = SYSTEM_IDLE;

signals.errorCode = 0;

signals.measurementCount = 0;

signals.lastMeasurementTime = 0;

}

void signal_manager_update(float distance) {

// Acquire mutex before updating shared signals

if (xSemaphoreTake(signalsMutex, pdMS_TO_TICKS(10)) == pdTRUE) {

signals.distance = distance;

signals.measurementCount++;

signals.lastMeasurementTime = xTaskGetTickCount();

// Update system state based on sensor data

updateSystemState(&signals);

// Release mutex

xSemaphoreGive(signalsMutex);

}

}

bool signal_manager_get_signals(SystemSignals *dest) {

bool success = false;

// Acquire mutex before reading shared signals

if (xSemaphoreTake(signalsMutex, pdMS_TO_TICKS(10)) == pdTRUE) {

// Copy signals to destination

memcpy(dest, &signals, sizeof(SystemSignals));

// Release mutex

xSemaphoreGive(signalsMutex);

success = true;

}

return success;

}

void acquisitionTask(void *pvParameters) {

TickType_t xLastWakeTime;

const TickType_t xFrequency = pdMS_TO_TICKS(100); // 100ms sampling rate

// Initialize the xLastWakeTime variable with the current time

xLastWakeTime = xTaskGetTickCount();

for (;;) {

// Wait for the next cycle with precise timing

vTaskDelayUntil(&xLastWakeTime, xFrequency);

// Read distance from sensor

float distance = sensor_read_distance();

// Update signals with new reading

signal_manager_update(distance);

}

}

- Sensor.cpp

/*

* sensor.cpp

*

* Implementation file for ultrasonic sensor functionality

*/

#include "sensor.h"

#include <NewPing.h>

// Global objects

static NewPing sonar(TRIGGER_PIN, ECHO_PIN, MAX_DISTANCE);

void sensor_init() {

// HC-SR04 requires no special initialization

// Pins are configured by the NewPing library

}

float sensor_read_distance() {

// Measure distance

float distance = sonar.ping_cm();

// If distance is 0, sensor might be out of range or there's an error

if (distance == 0) {

distance = MAX_DISTANCE; // Set to max distance if no echo received

}

return distance;

}

- Display.cpp

/*

* display.cpp

*

* Implementation file for display functionality

*/

#include "display.h"

#include <stdio.h>

void printReport(const SystemSignals *signals) {

// Print header

printf("\n========== SYSTEM REPORT ==========\n");

// Print current time

printf("Time: %lu ms\n", (unsigned long)xTaskGetTickCount());

// Print system state

printf("State: ");

switch (signals->state) {

case SYSTEM_IDLE:

printf("IDLE\n");

break;

case SYSTEM_MEASURING:

printf("MEASURING\n");

break;

case SYSTEM_ERROR:

printf("ERROR\n");

break;

default:

printf("UNKNOWN\n");

}

// Print distance

printf("Distance: %.2f cm\n", signals->distance);

// Print measurement count

printf("Measurements: %lu\n", (unsigned long)signals->measurementCount);

// Print error code if any

if (signals->errorCode > 0) {

printf("Error Code: %d\n", signals->errorCode);

}

printf("====================================\n");

}

void displayTask(void *pvParameters) {

// Add a small delay to offset from acquisition task

vTaskDelay(pdMS_TO_TICKS(50));

const TickType_t xFrequency = pdMS_TO_TICKS(500); // 500ms update rate

SystemSignals localSignals;

for (;;) {

// Get a copy of the current signals

if (signal_manager_get_signals(&localSignals)) {

// Print report using printf

printReport(&localSignals);

}

// Wait for the next cycle

vTaskDelay(xFrequency);

}

}

- Signal_manager.h

/*

* signal_manager.h

*

* Header file for signal and state management

*/

#ifndef SIGNAL_MANAGER_H

#define SIGNAL_MANAGER_H

#include <Arduino.h>

#include <Arduino_FreeRTOS.h>

#include <semphr.h>

// System state definitions

typedef enum {

SYSTEM_IDLE,

SYSTEM_MEASURING,

SYSTEM_ERROR

} SystemState;

// Signal structure to hold sensor data and system state

typedef struct {

float distance;         // Distance measured by ultrasonic sensor in cm

SystemState state;      // Current system state

uint8_t errorCode;      // Error code (0 = no error)

uint32_t measurementCount; // Count of measurements taken

TickType_t lastMeasurementTime; // Time of last measurement

} SystemSignals;

/**

* Initialize the signal manager

*/

void signal_manager_init();

/**

* Update system signals with new sensor data

*

* @param distance the newly measured distance

*/

void signal_manager_update(float distance);

/**

* Get a copy of the current system signals

*

* @param dest pointer to a SystemSignals struct to be filled with current values

* @return true if successful, false if mutex could not be acquired

*/

bool signal_manager_get_signals(SystemSignals *dest);

/**

* Task for acquiring data from the ultrasonic sensor

* Uses vTaskDelayUntil for precise timing

*/

void acquisitionTask(void *pvParameters);

#endif // SIGNAL_MANAGER_H

- Sensor.h

/*

* sensor.h

*

* Header file for ultrasonic sensor functionality

*/

#ifndef SENSOR_H

#define SENSOR_H

#include <Arduino.h>

// Pin definitions for HC-SR04 ultrasonic sensor

#define TRIGGER_PIN 9

#define ECHO_PIN 10

#define MAX_DISTANCE 400 // Maximum distance to measure (in cm)

/**

* Initialize the ultrasonic sensor

*/

void sensor_init();

/**

* Read distance from ultrasonic sensor

*

* @return measured distance in cm, or MAX_DISTANCE if no echo received

*/

float sensor_read_distance();

#endif // SENSOR_H

- Display.h

/*

* display.h

*

* Header file for display functionality

*/

#ifndef DISPLAY_H

#define DISPLAY_H

#include "signal_manager.h"

/**

* Print formatted report using printf

*

* @param signals pointer to the system signals to be displayed

*/

void printReport(const SystemSignals *signals);

/**

* Task for displaying data on Serial

* Uses vTaskDelay for timing

*/

void displayTask(void *pvParameters);

#endif // DISPLAY_H