Ministry of Education and Research of the Republic of Moldova

Technical University of Moldova

Department of Software and Automation Engineering

REPORT

Laboratory work No. 3.2

Discipline: Embedded Systems

Elaborated:   Berco Andrei, FAF - 221

Checked:	asist. univ., Martiniuc A.

Chișinău 2025

Analysis of the Situation in the Field

- Description of the Technologies Used and Application Context

Embedded systems are pivotal in real-time data acquisition and processing, enabling applications like environmental monitoring, industrial automation, and IoT. This lab work demonstrates:

- Signal conditioning for analog/digital sensors (e.g., noise filtering, scaling).

- FreeRTOS for deterministic task scheduling (e.g., vTaskDelayUntil for precise sampling).

- STDIO (via printf) for reporting data to LCD/Serial.

Application Context:

- Industrial IoT: Monitor temperature/humidity with filtered, real-time data.

- Smart Agriculture: Process soil moisture sensor data with adaptive filtering.

Overview of the Hardware and Software Components Used

System Architecture Explanation and Solution Justification

Task Breakdown

- Task_SensorRead

  - Reads ADC (analog) or I2C (digital) at fixed intervals using vTaskDelayUntil().

  - Example: adc_val = analogRead(SENSOR_PIN);

- Task_FilterData

  - Salt-and-Pepper Filter: Removes outliers (e.g., if (value > threshold) discard).

  - Weighted Moving Average: Smooths data (e.g., filtered = (0.6*current + 0.4*previous)).

- Task_Display

  - Converts ADC to voltage: Vout = (adc_val / 4095) * 3.3V.

  - Scales to physical units (e.g., °C for LM35: Temp = Vout * 100).

  - Outputs via printf("Temp: %.2f°C\n", temp); every 500ms.

Why FreeRTOS?

- Precision: Fixed sampling intervals avoid jitter.

- Modularity: Isolate sensor read, processing, and display tasks.

- Scalability: Add tasks (e.g., wireless transmission) without disrupting timing.

- Case Study: Sequential Task Execution in Embedded Systems

#### Context and Necessity

#### Many embedded applications (e.g., weather stations, smart agriculture) require periodic sensor readings with minimal latency. This case study demonstrates a FreeRTOS-based approach for reliable signal acquisition

#### Practical Implementation

- Task 1: Sensor Read

  - Analog sensors: analogRead() → scaling (e.g., Vout = (ADC_val / 4095) * 3.3V).

  - Digital sensors: I2C/SPI communication (e.g., BME280.readTemperature()).

  - Uses vTaskDelayUntil() for consistent sampling intervals.

- Task 2: Data Processing

  - Applies calibration/formulas (e.g., Temp = (Vo * 100) / 1024).

  - Stores results in a global struct for other tasks.

- Task 3: Display & Reporting

  - Prints formatted data via printf() every 500ms:

#### Extending the Case Study

- Wireless Transmission – Integrate Wi-Fi (ESP32) to send data to a cloud dashboard.

- Multi-Sensor Fusion – Add more sensors (CO₂, pressure) with independent tasks.

- Energy Optimization – Use vTaskSuspend() in battery-powered scenarios.

#### Benefits and Impact

- Structured Timing – FreeRTOS ensures no task starvation.

- Scalability – Easy to add more sensors/tasks.

- Debugging-Friendly – STDIO provides real-time logs.Scalability – Can be integrated with wireless communication, IoT frameworks, or real-time monitoring systems.

This case study highlights the importance of structured scheduling in embedded applications, demonstrating how sequential execution models enhance reliability and efficiency in task management.

Design

- Architectural Sketch and Component Interconnection

The system consists of:

- MCU (ESP32/Arduino) – Core processing unit.

- Sensor – Connected via:

- Analog: Direct to ADC pin.

- Digital: I2C (SCL/SDA) or SPI (MISO/MOSI/SCK).

- Serial Monitor – For debugging (printf output).

Figure 2.1 Component diagram

Figure 2.2 Component scheme

Figure 2.3 Electrical scheme

- Scheme bloc and algorithm

To understand the system's behavior, a Flowchart and a Finite State Machine (FSM) are used.

Flowchart – Serial Command Processing
(A Flowchart diagram illustrating the cycle: command reception → processing → execution → user feedback)

Figure 2.4 Flowchart

Explanation:

- Initialization: Sets up hardware (Serial, Ultrasonic sensor) and FreeRTOS (tasks, mutex).

- Ultrasonic Task (Cyclic):

  - Measures distance → Filters (salt & pepper → weighted average) → Saturates → Prints.

  - Uses vTaskDelayUntil() for precise timing.

Figure 2.4 FSM diagram

- Initialization: Configures pins and buffers.

- Idle:

  - WaitTrigger: Waits to send ultrasonic pulse.

  - MeasureDistance: Captures echo duration.

- FilterData:

  - SaltPepperFilter: Removes outliers via median.

  - WeightedAverage: Smooths with weighted window.

- Saturation: Ensures value is within [MIN_DISTANCE_CM, MAX_DISTANCE_CM].

- Output: Prints via printf (protected by mutex).

- Modular implementation

For better project organization, a modular architecture was used, dividing functionalities into separate files.

Figure 3.1 Project organization

The project is well-structured, separating functionality into different modules using a layered approach. The organization follows best practices for embedded systems and modular programming.

#### Code Functionality Overview:

- main.cpp

Figure 3.1: Flowchart for main.cpp

Description

- Purpose: Coordinates tasks using FreeRTOS.

- Key Sections:

  - Setup:

    - Initializes Serial, sensor, and filter buffers.

    - Creates a mutex to protect printf.

  - FreeRTOS Task (ultrasonic_distance_task):

    - Cyclic Workflow:

      - Read sensor → Filter → Saturate → Print.

      - Uses vTaskDelayUntil() for precise 100ms intervals.

  - Filter Pipeline:

    - Raw → Salt & Pepper → Weighted Average → Saturation.

- Critical Components:

  - data_mutex: Prevents Serial corruption during printf.

  - STACK_SIZE: Reduced to 128 bytes (Arduino Uno memory limits).

- filter.cpp

Figure 3.2: Flowchart for filter.cpp

Description

- Purpose: Implements digital filters for signal conditioning.

- Functions:

  - apply_salt_pepper_filter():

    - Shifts a sliding window buffer.

    - Sorts values (bubble sort) and returns the median.

  - apply_weighted_average_filter():

    - Applies user-defined weights to buffer values.

    - Returns the weighted sum.

  - apply_saturation():

    - Clamps values to a valid range (e.g., 0–500 cm).

- Key Variables:

  - buffer[]: Stores historical values for filtering.

  - weights[]: Predefined weights for averaging (e.g., [0.1, 0.2, 0.4, 0.2, 0.1]).

- Printf_redirect.cpp

Figure 3.4: Flowchart for printf_redirect	.cpp

Description

- Purpose: Redirects printf output to Arduino Serial.

- Functions:

  - init_printf_redirect():

    - Calls fdevopen() to link printf to serial_putchar.

  - serial_putchar():

    - Low-level function to write a character to Serial.

  - serial_printf() (fallback):

    - Formats strings using vsnprintf and sends via Serial.print().

- ultrasonic.cpp

Figure 3.5: Flowchart for ultrasonic.cpp

Description

- Purpose: Measures distance using an ultrasonic sensor (HC-SR04).

- Functions:

  - setup_ultrasonic_sensor():

    - Configures TRIGGER_PIN (output) and ECHO_PIN (input).

  - measure_distance():

    - Sends a 10µs trigger pulse.

    - Measures echo duration with pulseIn().

    - Calculates distance: distance = (pulse_duration × SOUND_SPEED) / 2.

- Key Constants:

  - SOUND_SPEED: 0.0343 cm/µs (speed of sound).

  - MAX_DISTANCE_CM: Timeout threshold (e.g., 30ms → ~500 cm).

- Results

- Figure 3.6: Physical result

Figure 3.7: Console result

Conclusions

After completing this laboratory work, the following results were achieved:

A functional sensor data acquisition and display system was successfully implemented, enabling real-time measurement, signal processing, and reporting using FreeRTOS and STDIO.

- The application correctly acquires ultrasonic sensor data, processes it into meaningful measurements (distance in cm), and displays the results on Serial and/or LCD at fixed intervals.

- A modular architecture was employed, separating functionalities into distinct tasks: sensor acquisition (100ms), signal management (mutex-protected), and display (500ms).

- The system was tested on a microcontroller platform (e.g., Arduino with FreeRTOS) and validated for real-time performance and data accuracy.

- Error handling was integrated to detect invalid sensor readings (e.g., negative distances), triggering an error state for robustness.

System Performance Analysis

The system demonstrated stable and efficient operation, with reliable sensor sampling and reporting. Key performance aspects include:

- Timing Precision:

  - Sensor data is acquired every 100ms using vTaskDelayUntil() for minimal jitter.

  - Display updates occur every 500ms without blocking acquisition.

- Data Integrity:

  - Shared signals (e.g., distance, state) are protected by a mutex, preventing race conditions.

  - Invalid readings (e.g., out-of-range values) automatically trigger an error state.

- Scalability:

  - The modular design allows easy integration of additional sensors (e.g., temperature, humidity) or output devices (e.g., Wi-Fi modules).

Identified Limitations

Despite its functionality, the system has areas for improvement:

- Sensor Error Recovery – The system detects errors but lacks an auto-recovery mechanism (e.g., retry logic).

- Limited Output Channels – Data is only displayed on Serial; adding LCD or wireless (Wi-Fi/MQTT) would enhance usability.

- Fixed Sampling Rate – The 100ms interval is hardcoded; a user-configurable rate (e.g., via buttons) would improve flexibility.

- Power Efficiency – No low-power modes are implemented, which could be critical for battery-operated deployments.

Impact of the Technology in Real-World Applications

This project demonstrates principles applicable to:

- Industrial Monitoring:

  - Ultrasonic sensors measure tank levels or object proximity, with FreeRTOS ensuring real-time data logging.

- Smart Agriculture:

  - Soil moisture or light sensors could replace the ultrasonic module, with data transmitted to a cloud dashboard.

- Home Automation:

  - The system could be extended to control actuators (e.g., lights, alarms) based on sensor thresholds.

- IoT Prototyping:

  - Adding an ESP32 Wi-Fi module would enable remote monitoring via MQTT/HTTP.

Improvement Suggestions

To enhance the system, consider:

- Dynamic Sampling Rates – Allow runtime adjustment (e.g., via Serial commands) for adaptive sensing.

- Wireless Integration – Use ESP-NOW or LoRa for remote data transmission.

- Advanced Error Handling – Implement sensor calibration or auto-retry on failure.

- Power Optimization – Integrate FreeRTOS tickless idle mode for battery savings.

- Multi-Sensor Support – Expand the signal manager to handle multiple sensor inputs (e.g., temperature + distance).

By addressing these points, the system could evolve into a versatile embedded platform for real-world IoT and automation applications.

Final Remarks

This lab successfully achieved its objectives, providing a foundation for real-time sensor systems with FreeRTOS. The modular design, mutex-protected data, and structured reporting make it a reproducible template for future embedded projects.

Next Steps:

- Integrate Wi-Fi (ESP32) for cloud connectivity.

- Develop a GUI dashboard (e.g., Node-RED) for visualization.

- Benchmark power consumption and optimize for low-energy scenarios.

### Note on AI Tool Usage

During the drafting of this report, the author used ChatGPT for generating and structuring the content. The resulting information was reviewed, validated, and adjusted according to the requirements of the laboratory work, ensuring technical accuracy and clarity of explanations.
The use of this AI tool was aimed at structuring and optimizing the presentation of information without replacing personal analysis and understanding of the subject.

Bibliography

- Official Arduino Documentation

  - Arduino Reference – Serial Communication
https://www.arduino.cc/reference/en/#communication

  - Arduino Mega 1280 Pinout & Datasheet
https://docs.arduino.cc/hardware/mega-1280

- PlatformIO Official Documentation

  - PlatformIO for Arduino Development
https://docs.platformio.org/en/latest/platforms/atmelavr.html

- TUM Courses

  - Introducere în Sistemele Embedded și Programarea Microcontrolerelor

  - Principiile comunicației seriale și utilizarea interfeței UART

Appendix

- GitHub: https://github.com/KaBoomKaBoom/ES_Labs.git

- Main.cpp

/**

* Ultrasonic Sensor Application with FreeRTOS for Arduino Uno

* Main file: Contains setup, loop and main task

*/

#include <Arduino.h>

#include <Arduino_FreeRTOS.h>

#include <semphr.h>

#include <stdio.h>

#include "ultrasonic.h"

#include "filter.h"

#include "printf_redirect.h"

// Task parameters

#define STACK_SIZE          128     // Reduced stack size for Arduino Uno

#define ACQUISITION_PERIOD  100     // Acquisition period in milliseconds

// Signal conditioning parameters

#define SALT_PEPPER_WINDOW  5       // Window size for salt and pepper filter

#define WEIGHTED_AVG_WINDOW 5       // Reduced window size for Arduino memory constraints

// Filter buffers

static float salt_pepper_buffer[SALT_PEPPER_WINDOW];

static float weighted_avg_buffer[WEIGHTED_AVG_WINDOW];

// Weights for weighted average (must sum to 1.0)

static const float weights[WEIGHTED_AVG_WINDOW] = {0.1, 0.2, 0.4, 0.2, 0.1};

// Function prototypes

void ultrasonic_distance_task(void *pvParameters);

// Mutex for data access protection

SemaphoreHandle_t data_mutex;

void setup() {

// Initialize serial communication

Serial.begin(115200);

while (!Serial) {

; // Wait for serial port to connect

}

// Initialize printf redirection

init_printf_redirect();

printf("Ultrasonic Sensor Application Starting...\n");

// Create mutex

data_mutex = xSemaphoreCreateMutex();

// Initialize hardware

setup_ultrasonic_sensor();

// Initialize filter buffers

for (int i = 0; i < SALT_PEPPER_WINDOW; i++) {

salt_pepper_buffer[i] = 0.0;

}

for (int i = 0; i < WEIGHTED_AVG_WINDOW; i++) {

weighted_avg_buffer[i] = 0.0;

}

// Create task

xTaskCreate(

ultrasonic_distance_task,       // Task function

"ultrasonic",                   // Task name

STACK_SIZE,                     // Stack size

NULL,                           // Parameters

1,                              // Priority

NULL                            // Task handle

);

printf("Application initialized successfully\n");

// Start the scheduler

vTaskStartScheduler();

}

void loop() {

// Empty. Things are done in Tasks.

}

/**

* Task for ultrasonic distance measurement and processing

*/

void ultrasonic_distance_task(void *pvParameters) {

(void) pvParameters;

TickType_t last_wake_time;

const TickType_t period = pdMS_TO_TICKS(ACQUISITION_PERIOD);

// Initialize the last_wake_time variable with the current time

last_wake_time = xTaskGetTickCount();

float raw_distance, filtered_distance1, filtered_distance2, final_distance;

while (1) {

// Get raw distance measurement

raw_distance = measure_distance();

// Apply salt and pepper filter

filtered_distance1 = apply_salt_pepper_filter(raw_distance, salt_pepper_buffer, SALT_PEPPER_WINDOW);

// Apply weighted average filter

filtered_distance2 = apply_weighted_average_filter(filtered_distance1, weighted_avg_buffer, weights, WEIGHTED_AVG_WINDOW);

// Apply saturation to keep values in valid range

final_distance = apply_saturation(filtered_distance2, MIN_DISTANCE_CM, MAX_DISTANCE_CM);

// Acquire mutex to print data

if (xSemaphoreTake(data_mutex, portMAX_DELAY) == pdTRUE) {

// Print results using printf

printf("Distance Measurements:\n");

printf("  Raw: %.2f cm\n", raw_distance);

printf("  After Salt & Pepper: %.2f cm\n", filtered_distance1);

printf("  After Weighted Avg: %.2f cm\n", filtered_distance2);

printf("  Final (Saturated): %.2f cm\n\n", final_distance);

// Release mutex

xSemaphoreGive(data_mutex);

}

// Wait precisely for the next cycle using vTaskDelayUntil

vTaskDelayUntil(&last_wake_time, period);

}

}

- filter.cpp

/**

* Digital filters module implementation file

*/

#include <Arduino.h>

#include <string.h>

#include "filter.h"

/**

* Salt and Pepper filter (median filter)

* Removes impulsive noise by taking the median value from a window

*/

float apply_salt_pepper_filter(float new_value, float *buffer, int window_size) {

float sorted_values[window_size];

// Shift values in buffer

for (int i = 0; i < window_size - 1; i++) {

buffer[i] = buffer[i + 1];

}

buffer[window_size - 1] = new_value;

// Copy values to sorting array

memcpy(sorted_values, buffer, sizeof(float) * window_size);

// Perform simple bubble sort (sufficient for small arrays)

for (int i = 0; i < window_size - 1; i++) {

for (int j = 0; j < window_size - i - 1; j++) {

if (sorted_values[j] > sorted_values[j + 1]) {

float temp = sorted_values[j];

sorted_values[j] = sorted_values[j + 1];

sorted_values[j + 1] = temp;

}

}

}

// Return median value

return sorted_values[window_size / 2];

}

/**

* Weighted Average filter

* Applies weighted average to smooth the signal

*/

float apply_weighted_average_filter(float new_value, float *buffer, const float *weights, int window_size) {

// Shift values in buffer

for (int i = 0; i < window_size - 1; i++) {

buffer[i] = buffer[i + 1];

}

buffer[window_size - 1] = new_value;

// Apply weighted average

float weighted_sum = 0.0;

for (int i = 0; i < window_size; i++) {

weighted_sum += buffer[i] * weights[i];

}

return weighted_sum;

}

/**

* Apply saturation to ensure value is within valid range

*/

float apply_saturation(float value, float min_val, float max_val) {

if (value < min_val) {

return min_val;

} else if (value > max_val) {

return max_val;

} else {

return value;

}

}

- ultrasonic.cpp

/**

* Ultrasonic sensor module implementation file

*/

#include <Arduino.h>

#include "ultrasonic.h"

/**

* Setup function for ultrasonic sensor pins

*/

void setup_ultrasonic_sensor(void) {

// Configure trigger pin as output

pinMode(TRIGGER_PIN, OUTPUT);

digitalWrite(TRIGGER_PIN, LOW);

// Configure echo pin as input

pinMode(ECHO_PIN, INPUT);

}

/**

* Measure distance using ultrasonic sensor

* @return Distance in centimeters

*/

float measure_distance(void) {

// Variables to store pulse timing

unsigned long pulse_start, pulse_end;

float pulse_duration, distance;

// Clear trigger pin

digitalWrite(TRIGGER_PIN, LOW);

delayMicroseconds(2);

// Send a 10µs pulse to trigger

digitalWrite(TRIGGER_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIGGER_PIN, LOW);

// Measure the length of echo pulse

pulse_duration = pulseIn(ECHO_PIN, HIGH, 30000); // Timeout after 30ms

// If timeout occurred, return max distance

if (pulse_duration == 0) {

return MAX_DISTANCE_CM;

}

// Calculate distance: pulse duration * speed of sound / 2

// (divided by 2 because sound travels to object and back)

distance = (pulse_duration * SOUND_SPEED) / 2.0;

return distance;

}

- printf_redirect.cpp

/**

* Printf redirection module implementation file

*/

#include <Arduino.h>

#include <stdio.h>

#include <stdarg.h>

#include "printf_redirect.h"

/**

* Initialize printf redirection to Serial

*/

void init_printf_redirect(void) {

// Redirect stdout to Serial

fdevopen(&serial_putchar, NULL);

}

/**

* Implementation of putchar for printf

*/

int serial_putchar(char c, FILE *stream) {

Serial.write(c);

return 0;

}

/**

* Custom implementation for printf that redirects to Serial

* This is a backup method if fdevopen doesn't work

*/

int serial_printf(const char *format, ...) {

char buf[128]; // Buffer for formatted string

va_list args;

va_start(args, format);

vsnprintf(buf, sizeof(buf), format, args);

va_end(args);

Serial.print(buf);

return strlen(buf);

}